DIGITAL LOGIC DESIGN PRACTICAL

SESSION 3: HALF ADDER AND FULL ADDER

# AIM

To build the Half Adder and Full Adder chips in Nand2Tetris HDL, understand their sum and carry generation, trace carry propagation for a sample input, and verify the outputs using the Nand2Tetris Hardware Simulator.

# HALF ADDER

## Boolean Expressions

Sum = A ⊕ B
Carry = A · B

## Truth Table

| A | B | Sum | Carry |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

## HDL CODE

HalfAdder

CHIP HalfAdder { IN a, b; // 1-bit inputs OUT sum, // Right bit of a + b carry; // Left bit of a + b PARTS: Xor(a=a, b=b, out=sum); And(a=a, b=b, out=carry); }

## SIMULATION OUTPUT

Figure 1: HalfAdder HDL implementation and successful Hardware Simulator output.

# FULL ADDER

## Boolean Expressions

Sum = A ⊕ B ⊕ C
Carry = AB + AC + BC

## Truth Table

| A | B | C | Sum | Carry |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

## HDL CODE

FullAdder

CHIP FullAdder { IN a, b, c; OUT sum, carry; PARTS: Xor(a=a, b=b, out=ab); Xor(a=ab, b=c, out=sum); And(a=a, b=b, out=c1); And(a=a, b=c, out=c2); And(a=b, b=c, out=c3); Or(a=c1, b=c2, out=c12); Or(a=c12, b=c3, out=carry); }

## CARRY PROPAGATION — SAMPLE INPUT

Sample input: A = 1, B = 1, C = 0

| Stage | Operation | Output |
| --- | --- | --- |
| 1 | A ⊕ B | 1 ⊕ 1 = 0 → ab = 0 |
| 2 | ab ⊕ C | 0 ⊕ 0 = 0 → Sum = 0 |
| 3 | A · B | 1 · 1 = 1 → c1 = 1 |
| 4 | A · C | 1 · 0 = 0 → c2 = 0 |
| 5 | B · C | 1 · 0 = 0 → c3 = 0 |
| 6 | c1 + c2 | 1 + 0 = 1 → c12 = 1 |
| 7 | c12 + c3 | 1 + 0 = 1 → Carry = 1 |

Final output: Sum = 0, Carry = 1. The carry generated by A and B propagates through c1 → c12 → carry.

## SIMULATION OUTPUT

Figure 2: FullAdder HDL implementation and successful Hardware Simulator output.

# RESULT

The Half Adder and Full Adder chips were implemented successfully using Nand2Tetris HDL. The Half Adder generates the sum and carry for two 1-bit inputs, while the Full Adder incorporates an additional carry input and produces the corresponding sum and carry output. Carry propagation was traced for a sample input, and both circuits were successfully verified using the Nand2Tetris Hardware Simulator.