DIGITAL LOGIC DESIGN PRACTICAL

AIM

Implement XOR, Mux (2-way, 4-way), and DMux chips in HDL.

XOR chip

BOOLEAN EXPRESSION

out = a'b + ab'

MUX chip(2-way)

BOOLEAN EXPRESSION

out = a·sel' + b·sel

DMux chip

BOOLEAN EXPRESSION

a = in·sel'

b = in·sel

HDL CODE

XOR chip

CHIP Xor {
    IN a, b;
    OUT out;

    PARTS:
    Nand(a=a, b=a, out=x);
    Nand(a=b, b=b, out=y);
    Nand(a=a, b=y, out=z);
    Nand(a=b, b=x, out=z1);

Nand(a=z, b=z1, out=out);

}

Mux chip (2-Way)

CHIP Mux {
    IN a, b, sel;
    OUT out;

    PARTS:
    Not(in=s, out=n);
    And(I0= I0, I1=nsel, out=x);
    And(I0=b, I1=sel, out=y);
    Or(I0=x, L1=y, out=out);
}

DMux chip

CHIP DMux {
    IN in, sel;
    OUT a, b;

    PARTS:
    Not(in=sel, out=nsel);
    And(a=in, b=nsel, out=a);
    And(a=in, b=sel, out=b);
}

SIMULATION OUTPUT

XOR chip

Mux chip (2-Way)

DMux chip

RESULT

The XOR, 2-Way Mux, Mux4Way16, and DMux chips were implemented successfully using HDL. The outputs can be verified using the Nand2Tetris Hardware Simulator and compared with the expected truth tables and test scripts.