DIGITAL LOGIC DESIGN PRACTICAL

AIM

To implement NOT, AND and OR gates using NAND gate using the NAND2TETRIS Simulator.

Truth Table

| A | Y |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

Truth Table

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Truth Table

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

HDL CODE

NOT

CHIP Not {
    IN in;
    OUT out;

    PARTS:
    Nand(a=in, b=in, out=out);
}

AND

CHIP And {
    IN a, b;
    OUT out;

    PARTS:
    Nand(a=a, b=b, out=nandOut);
    Nand(a=nandOut, b=nandOut, out=out);
}

OR

CHIP Or {
    IN a, b;
    OUT out;

    PARTS:
    Nand(a=a, b=a, out=notA);
    Nand(a=b, b=b, out=notB);
    Nand(a=notA, b=notB, out=out);
}

SIMULATION OUTPUT

NOT

AND

OR

RESULT

The NOT, AND and OR gates were implemented successfully using only NAND gates. The outputs can be checked in the NAND2TETRIS Simulator and compared with the expected truth tables.